Annex 4 – Goods and services specification

The contractor shall install 8 CCTV camera along with supporting accessories in British Council, Lainchaur as per the specification below.

| S. No. | Description | Units | Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Supply of Bosch or Honeywell or Equivalent Indoor Dome Camera NII-50022-A3 Supply of 1080p Indoor Dome camera with 1/2.7-inch CMOS, Total sensor pixels 1952 x 1092 (2MP), Sensitivity – (3200K, reflectivity 89%, F1.3, 30IRE) Color 0.24 lx, Mono 0.05 lx, With IR 0.0 lx, Dynamic range, Fully configurable quad streaming, auto zoom/focus lens, Regions of interest and E-PTZ, White Balance, Automatic Electronic Shutter (AES);Backlight compensation, Intelligent Dynamic Noise Reduction, Intelligent defog, Privacy Masking, MOTION+, 3 to 10 mm Varifocal lens, Alarm input/output- 1/1, Audio Streaming Full-duplex / half duplex, Signal-to-Noise Ratio >50 dB, SD card slot, ONVIF Profile S, 3-axis adjustment, CE, UL, FCC certified. | Unit | 7.00 |  |
| 2 | Supply of Bosch or Honeywell or Equivalent NBE-3502-AL 5MP IR Bullet camera with 1/2.9-inch CMOS, Total sensor pixels 3072 (H) x 1728 (V); up to 25fps; Sensitivity – Measured according to IEC 62676 Part 5 (1/30, F1.6), Color 0.379 lx, Mono 0.042 lx, With IR 0.0 lx, 128dB WDR, H.265; H.264; M- JPEG, Automatic Electronic Shutter (AES); Built-in Essential Video Analytics to trigger relevant alerts and quickly retrieve data Intelligent Dynamic Noise Reduction, Intelligent Defog,Rule based alarms and tracking, Line crossing, Enter / leave field, Follow route, Loitering, Idle / removed object, People counting, Crowd density estimation, 3D tracking based on Duration, Size, Aspect ratio v/h, Speed, Direction, Color, Object classes (Upright persons, Bikes, Cars, Trucks), Privacy Masking, Pixel counter, Memory card slot, IR Range 30mtr, Automatic Varifocal 3.2 to 10 mm, Alarm Input 1/1, Audio Input/output, RJ45, G.711, Audio Streaming Full-duplex / half duplex, ONVIF Profile S, G & T, IP66, IK10, PoE, CE, FCC, UL certified. SD card support up to 2TB. FTP, Dropbox and e-mail applications | Unit | 1.00 | This camera is to be installed outdoors and should have IP66 rating |
| 3 | Wiring installation testing with pure copper cat 6 with PVC conduct pipe and required other accessories | Job | 1.00 | Existing conduits and cables to be used where feasible after testing which can be done during site visit |
| 4 | Supply of Bosch or Honeywell or Equivalent Network video recorder 32channel DRN-5532-414N00 4SATA port 10TB each port maximum (standard configuration), all-in-one video management solution for up to 32 channels Out-of-the-box IP video management solution with 10 TB storage capacity Robust, secure operation - instant real time access to video Instantly find what you are looking for Advanced user and alarm management 24TB HDD included | Job | 1.00 | This system is to be linked with the existing system details to be provided during site visit. |
| 5 | Supply installation testing of 24 port PoE switch - Layer 1 24-port 10/100/1000T | Job | 1.00 |  |